Computer Architecture Lab

(EL-2012)

LABORATORY REPORT

Fall 2026

(Lab#02)

Write Lab Title Here

Student Name: Muhammad Hamza  Roll No: 24i-6513   Section: A

Date performed: 02 , Sep , 2026

MARKS AWARDED: ________ / 10

Signature and date of Instructor: _________________

Lab Instructors: Dr. Nasir Ali Shah, Engr. Fasih Ahmad

______________________________________________________________________________________________________________________________________________________

NATIONAL UNIVERSITY OF COMPUTER AND EMERGING SCIENCES, ISLAMABAD

| Prepared by: | Fasih Ahmad |  | Version: 1.0 |
| --- | --- | --- | --- |
| Verified by: | Dr. Nasir Ali Shah |  | Updated: Fall 2026 |

Task 1:

Description:

// Write description of the task here 👇 (max. five to seven lines) In this task we created half adder using different level of abstraction like using (Gate Level modeling , Dataflow Level modeling and Behavioral modeling) and learned how this design are different from each other.

Code:

//Half Adder Gate Level Modeling module Half_Adder(a, b , s , c); input a , b; output s , c; and a1(c , a, b); xor x1(s , a , b); endmodule module testbench(); wire sum , carry; reg in1 ; reg in2 ; Half_Adder uut(in1, in2 , sum , carry); initial begin in1= 0 ; in2 = 0; #50; in1= 0 ; in2 = 1; #50; in1= 1 ; in2 = 0; #50; in1= 1 ; in2 = 1; #50; end endmodule; //Half Adder Date Flow Level modeling module Half_Adder(A , B , S , C); input A , B; output S , C; assign S = A^B; assign C = A&B; endmodule module TestBench_Half_Adder(); reg in1 , in2 ; wire sum , carry; Half_Adder hd(in1 ,in2 , sum ,carry); integer i; initial begin for (i = 0 ; i < 4 ;i = i+1) begin {in1 , in2}= i; #50; end end endmodule; // Behavioral Level modeling module Half_Adder(A , B , S , C); input A , B; output reg S , C; always @(A or B) begin S = A^B; C = A&B; end endmodule // how are you module TestBench_Half_Adder_Behavioral(); reg in1 , in2 ; wire sum , carry; Half_Adder hd(in1 ,in2 , sum ,carry); integer i; initial begin for (i = 0 ; i < 4 ;i = i+1) begin {in1 , in2}= i; #50; end end endmodule. // Gate level model of full adder module Half_Adder(a, b , s , c); input a , b; output s , c; and a1(c , a, b); xor x1(s , a , b); endmodule module Full_Adder(x , y , cin , sum , carry) ; input x , y , cin; output sum , carry; wire t1 , t2 , t3 ; Half_Adder h1 (x, y , t1 , t2); Half_Adder h2(t1 , cin , sum , t3); or (carry , t3 , t2); endmodule module testbench(); wire sum , carry; reg in1 , in2 , in3; Full_Adder uut(in1, in2 , in3 , sum , carry); initial begin in1= 0 ; in2 = 0; in3 =0; #50; in1= 0 ; in2 = 0; in3 =1; #50; in1= 0 ; in2 = 1; in3 =0; #50; in1= 0 ; in2 = 1; in3 =1; #50; in1= 1 ; in2 = 0; in3 =0; #50; in1= 1 ; in2 = 0; in3 =1; #50; in1= 1 ; in2 = 1; in3 =0; #50; in1= 1 ; in2 = 1; in3 =1; #50; end endmodule; // Data Flow Modeling of Full Adder module Half_Adder(a, b , s , c); input a , b; output reg s , c; assign s = a^b; assign c = a&b; endmodule module Full_Adder(x , y , cin , sum , carry) ; input x , y , cin; output sum , carry; wire t1 , t2 , t3 ; Half_Adder h1 (x, y , t1 , t2); Half_Adder h2(t1 , cin , sum , t3); assign carry = t3 ¦ t2; endmodule module testbench_FullAdder_Dataflow(); wire sum , carry; reg in1 , in2 , in3; Full_Adder uut(in1, in2 , in3 , sum , carry); integer i; initial begin for (i = 0 ;i < 8 ; i= i+1) begin {in1 , in2 , in3 } = i; #50; end end endmodule; // Behavioral Modeling module Full_Adder(x , y , cin , sum , carry) ; input x , y , cin; output reg sum , carry; always @(*) begin if (x == 0 && y ==0 && cin == 0) begin sum =0 ; carry = 0; end else if (x == 0 && y ==0 && cin == 1) begin sum =1 ; carry = 0; end else if (x == 0 && y ==1 && cin == 0) begin sum =1 ; carry = 0; end else if (x == 0 && y ==1 && cin == 1) begin sum =0 ; carry = 1; end else if (x == 1 && y ==0 && cin == 0) begin sum =1 ; carry = 0; end else if (x == 1 && y ==0 && cin == 1) begin sum =0 ; carry = 1; end else if (x == 1 && y ==1 && cin == 0) begin sum =0 ; carry = 1; end else begin sum =1 ; carry = 1; end end endmodule module testbench_FullAdder_Behavioral(); wire sum , carry; reg in1 , in2 , in3; Full_Adder uut(in1, in2 , in3 , sum , carry); integer i; initial begin for (i = 0 ;i < 8 ; i= i+1) begin {in1 , in2 , in3 } = i; #50; end end endmodule.

Output:

Logism Half_Adder data flow Half adder Behavioral modeling Full Adder Data flow Full adder Behavioral

Task 2:

Description:

// Write description of the task here 👇 (max. five to seven lines) In this task I made a 3x8 decoder and it’s quit easy in the model sim. We used different approaches to design it like gate level and data flow level modeling.

Code:

// Paste source code here 👇 // 3x8 Decoder using Gate level modeling module Decoder(s0, s1, s2, D0, D1, D2, D3, D4, D5, D6, D7); input s0, s1, s2; output D0, D1, D2, D3, D4, D5, D6, D7; wire inv0, inv1, inv2; not (inv0, s0); not (inv1, s1); not (inv2, s2); and (D0, inv0, inv1, inv2); and (D1, inv0, inv1, s2); and (D2, inv0, s1, inv2); and (D3, inv0, s1, s2); and (D4, s0, inv1, inv2); and (D5, s0, inv1, s2); and (D6, s0, s1, inv2); and (D7, s0, s1, s2); endmodule module testbench_3x8Decoder(); reg in1 , in2 , in3; wire d0 , d1 , d2 , d3 , d4 , d5 , d6 , d7; Decoder uut(in1, in2 , in3 , d0 , d1 , d2 , d3 , d4 , d5 , d6 , d7); initial begin in1= 0 ; in2 = 0; in3 =0; #50 in1= 0 ; in2 = 0; in3 =1; #50 in1= 0 ; in2 = 1; in3 =0; #50 in1= 0 ; in2 = 1; in3 =1; #50 in1= 1 ; in2 = 0; in3 =0; #50 in1= 1 ; in2 = 0; in3 =1; #50 in1= 1 ; in2 = 1; in3 =0; #50 in1= 1 ; in2 = 1; in3 =1; end endmodule; // 3x8 Decoder using Data flow model module Decoder(s0, s1, s2, D0, D1, D2, D3, D4, D5, D6, D7); input s0, s1, s2; output D0, D1, D2, D3, D4, D5, D6, D7; assign D0 = !s0 & !s1 & !s2; assign D1 = !s0 & !s1 & s2; assign D2 = !s0 & s1 & !s2; assign D3 = !s0 & s1 & s2; assign D4 = s0 & !s1 & !s2; assign D5 = s0 & !s1 & s2; assign D6 = s0 & s1 & !s2; assign D7 = s0 & s1 & s2; endmodule module testbench_3x8Decoder_DataFlow(); reg in1 , in2 , in3; wire d0 , d1 , d2 , d3 , d4 , d5 , d6 , d7; Decoder uut(in1, in2 , in3 , d0 , d1 , d2 , d3 , d4 , d5 , d6 , d7); initial begin in1= 0 ; in2 = 0; in3 =0; #50; in1= 0 ; in2 = 0; in3 =1; #50; in1= 0 ; in2 = 1; in3 =0; #50; in1= 0 ; in2 = 1; in3 =1; #50; in1= 1 ; in2 = 0; in3 =0; #50; in1= 1 ; in2 = 0; in3 =1; #50; in1= 1 ; in2 = 1; in3 =0; #50; in1= 1 ; in2 = 1; in3 =1; end endmodule.

Output:

// Import output here 👇 Gate Level Modeling Dataflow modeling

Task 3:

| Description: |  |  |
| --- | --- | --- |
| // Write description of the task here 👇 (max. five to seven lines) In this task I made a 8to1 Mux which is basically a selector we have the choice to select the input by using the select lines so we made it by two method the gate level and data flow level modeling. |  |  |
| Code: |  |  |
| // Paste source code here 👇 module MUX_8to1 ( s0, s1, s2,i0, i1, i2, i3, i4, i5, i6, i7, out); input s0 , s1 , s2; input i0, i1, i2, i3, i4, i5, i6, i7; output out; wire inv0, inv1, inv2; not (inv0, s0); not (inv1, s1); not (inv2, s2); wire D0, D1, D2, D3, D4, D5, D6, D7; and (D0, i0, inv0, inv1, inv2); // 000 and (D1, i1, inv0, inv1, s2); // 001 and (D2, i2, inv0, s1, inv2); // 010 and (D3, i3, inv0, s1, s2); // 011 and (D4, i4, s0, inv1, inv2); // 100 and (D5, i5, s0, inv1, s2); // 101 and (D6, i6, s0, s1, inv2); // 110 and (D7, i7, s0, s1, s2); // 111 or (out, D0, D1, D2, D3, D4, D5, D6, D7); endmodule module TestBench_MuxGatelevel(); reg s0, s1 ,s2; reg i0, i1, i2, i3, i4, i5, i6, i7; wire Y; MUX_8to1 uut(s0,s1,s2 , i0, i1, i2, i3, i4, i5, i6, i7 , Y); integer i; initial begin i0 = 0; i1 = 1; i2 = 0; i3 = 1; i4 = 0; i5 = 1; i6 = 0; i7 = 1; for (i =0 ;i < 8; i = i+1) begin {s0 , s1 ,s2 } = i; #50; end end endmodule | // Data flow model module MUX_8to1 ( s0, s1, s2,i0, i1, i2, i3, i4, i5, i6, i7, out); input s0 , s1 , s2; input i0, i1, i2, i3, i4, i5, i6, i7; output out; assign out = (i0 & !s0 & !s1 & !s2) ¦ (i1 & !s0 & !s1 & s2) ¦ (i2 & !s0 & s1 & !s2) ¦ (i3 & !s0 & s1 & s2) ¦ (i4 & s0 & !s1 & !s2) ¦ (i5 & s0 & !s1 & s2) ¦ (i6 & s0 & s1 & !s2) ¦ (i7 & s0 & s1 & s2); endmodule module TestBench_MuxDataflow(); reg s0, s1 ,s2; reg i0, i1, i2, i3, i4, i5, i6, i7; wire Y; MUX_8to1 uut(s0,s1,s2 , i0, i1, i2, i3, i4, i5, i6, i7 , Y); integer i; initial begin i0 = 0; i1 = 1; i2 = 0; i3 = 1; i4 = 0; i5 = 1; i6 = 0; i7 = 1; for (i =0 ;i < 8; i = i+1) begin {s0 , s1 ,s2 } = i; #50; end end endmodule |
| Output: |  |  |
| // Import output here 👇 Gatelevel Model Dateflow model |  |  |